- DIY simple and low-cost potentiostat

There is an increasing DIY culture around instrumentational educational tools. Despite this positive trend there is only a small number of projects available publicly, what is describing a cheap and simple builds and methods for students and teachers.

With the following design a true low-cost and simple potentiostat is presented. Overall cost is around 10-14 $ to build the device. It is based on a small form factor Arduino clone: Seeeduino XIAO. The uContoller has a 10 bits D/A and 12 bits A/D converter. The project files contain the hardware design files:

- circuit diagram,

- bill of materials (list of components),

- gerber files.

You will also find the software components as well:

- Arduino sketch,

- Graphical User Interface (GUI).

Current version of the Arduino sketch describes the Cyclic Voltammetry (CV) measurement, but users can develop other voltametric technics (e.g. LSV, SW, CA) by expanding the Arduino sketch.

  - Bill of materials (List of components)

IC1 AD8648ARZ (SOIC)	1 db	845.- HUF

Seeeduino XIAO		1 db	1835.- HUF

IC socket (DIL14 )		1 db	30.- HUF

10 kΩ SMD 1206		5 db	45.- HUF

510 Ω SMD 1206	 	2 db	15.- HUF

100 nF SMD 1206 	1 db	110.- HUF

10 µF   SMD 1206                2 db  	120.- HUF

USB Type-C cable		1 db	700.- HUF

SUM:			3700.- HUF (~ 12 $)

  - Circuit diagram

  - Arduino Sketch

#define DAC_PIN A0   // DAC connection to Seeduino

int dacVolt=0;  // DAC variable;

float current=0 ; // current

float volt=0; // voltage

/*V(start,stop)=513+V*312 V=in Volts namely V(appl)=513+V*312

V(start)=205      equals to -1,00 Volt; V(stop)=825 equals to +1,00 Volt

V(start)=513      equals to 0,00 Volt;

S(rate)=60          (S(rate)= 300-60-30 equals to 10-50-100 mV/sec)

V(appl) max.value= +/-1,2 Volt

*/

int V_start=413; // V(start) the CV starting voltage

int V_stop=700; // V(stop) the CV ’turning’ voltage

int S_rate=30; // S(rate)= 300-60-30 equals to 10-50-100 mV/sec

void setup () {

analogWriteResolution(10);

analogReadResolution(12);

Serial.begin(9600);

}

void loop(){

if (Serial.available() > 0)

{

String myString = "";

myString = Serial.readString();

char ch = myString.charAt(0);

// Measurement cycle starting with  M command

if (ch=='M')

{

//Start forward scan

for(dacVolt=V_start; dacVolt<=V_stop; dacVolt++){

analogWrite(DAC_PIN,dacVolt);

delay(S_rate);

float current=analogRead(A1)*3.3/4096;

float volt=analogRead(A2)*(-3.3)/4096;

Serial.print(1*(volt+1.65));//potential in V

Serial.print(",");

Serial.print (100*(current-1.65));//current in µA

Serial.print(",");

Serial.println();

delay(S_rate);

}

//Start revers scan

for(dacVolt=V_stop; dacVolt>=V_start; dacVolt--){

analogWrite(DAC_PIN,dacVolt);

delay(S_rate);

float current=analogRead(A1)*3.3/4096;

float volt=analogRead(A2)*(-3.3)/4096;

Serial.print(1*(volt+1.65));//voltage in V

Serial.print(",");

Serial.print (100*(current-1.65));//current in mikroA

Serial.print(",");

Serial.println();

delay(S_rate);

}

Serial.println('#');

}

}

}

  - Screenshot of a real CV measuremet (SeeedSTAT.exe)

- LOAD EXAMPLE PROJECT

- Open COM Port

- START -> Begin new measurement.